HOME IMPROVEMENT PROJECT #1

Few things take as much daily abuse as your floors. They get scuffed and kicked, heavy stuff gets dropped on them, and people walk all over them all the time. No wonder they need occasional repair and replacement! Flooring also makes a major design statement for your house. The color, style, and type of flooring you chose is an important decision that you have to live with for years Flooring repair and installation can be big jobs. In some instances, specialty tools such as floor sanders, carpet stretchers, and tile-cutting saws will help make the work much faster and easier. You can rent these kinds of specialty power tools instead of buying them, too.

Stopping Squeaks

Floors and stairs squeak when wooden floorboards or structural elements rub against each other, when the bridging between joists flexes under traffic, or when floorboards have not been properly nailed to the subfloor. Fix squeaks from underneath the floor or staircase, if you can. If the underside is covered, you'll have to work from above. With hardwood floors, drive ring-shank or cement-coated flooring nails into the seams between boards. Separate wooden bridging members to eliminate noise problems. To quiet a squeaky floor covered with deep-pile carpet, drive a wallboard screw through the carpet and pad into the floor joist. Countersink the screw head in the subfloor. This releases any trapped pad under the screw head and allows the carpet to lie flat.

Stuff You’ll Need: Tools Drill, Hammer, Screwdriver, Nail set

Materials Wood shims, Ring-shank or cement-coated flooring nails, Screws, 1x4 lumber

There are 6 steps in this project:

- Shimming the subfloor: Wedge shims between the joist and subfloor and tap them into place

- Cleating the subfloor: Where several boards in the subfloor above a joist are moving, securing them with a cleat works better than shimming the boards individually. A piece of 1x4, wedged against the subfloor and nailed to the joist solves this problem.

- Reinforcing joists: Squeaking over a large area may indicate that the joists beneath the floor are shifting slightly and inadequately supporting the subfloor. Steel bridging, nailed between joists, keeps the joists from moving side to side and stabilizes the subfloor.

- Driving screws from below: Drill a pilot hole through the subfloor, then a smaller pilot hole into the finished floor.

- Nailing from above: When you can't get access to the floor from below, drill pilot holes and nail through the surface. Locate the floor joists and nail directly into them for a fastening job that won't work loose. Countersink the nail heads.

- Anchoring stair treads to risers: Driving flooring nails at opposing angles assures they won't come loose again. With hardwood treads, drill pilot holes for the nails, drive the nails into the risers, and countersink the nail heads. Fill the nail holes with wood putty.

Vendors: Chris Lemke Lumber, SafeCo Deck Systems